MEMORANDUM

| To: | Charlie Refvem, Department of Mechanical Engineering, Cal Poly SLO crefvem@calpoly.edu |
| --- | --- |
| From: | Colby Cordoba ccordoba@calpoly.edu Hobbs Hegedus hhegedus@calpoly.edu |
| Date: | 2-22-2026 |
| RE: | UI Design and Automated Data Collection |

# Assignment

In this lab, we were tasked with designing a serial-based user interface to tune and evaluate the closed-loop motor control system on the Romi robot. Building upon the PI motor controller developed in Lab 0x03, the system was extended to support automated step-response testing, runtime gain adjustment, and structured data logging for PC-side analysis.

The firmware is organized using a cooperative multitasking framework, where each subsystem (user interface, motor control, and data collection) runs as a scheduled task under the pri_sched() scheduler. Hardware initialization, shared variables, and inter-task queues are created in main.py, while all user interaction is handled by dedicated UI tasks to prevent blocking real-time motor and control execution.

Two user interface tasks are implemented: a tuning interface for entering controller gains and setpoints, and a data-collection interface for triggering step-response tests. The tuning interface accepts multi-character numeric input for proportional and integral gains as well as the desired speed setpoint. This input logic is adapted from the provided multichar_input.py example and integrated into the UI task to remain compatible with cooperative scheduling.

When step-response testing is initiated, control is passed to the data-collection interface, where the user can command left, right, or simultaneous motor tests, stop an active test, or return to the tuning menu. During testing, the UI is intentionally locked out while motor velocity data and timestamps are buffered using queues. Once the collection is complete, the data is printed in CSV format over the serial port.

After validating system operation using PuTTY, the interface is designed to be driven by a PC-side Python script. This script opens the serial port, sends the same single-character commands used during manual testing, captures the returned CSV data, saves it to timestamped files, and automatically generates step-response plots for analysis.

The file step_collector_loop.py automates step-response testing by acting as a “wrapper” around the interface. Instead of a human typing commands in PuTTY, the script opens the same serial port (COM9, 115200 baud) and transmits the same single-character commands and numeric entries. This allows repeated tests to be run consistently and data to be logged automatically for analysis and plotting.

# Script Workflow

- Serial connection + synchronization

  - The script opens the serial port with a short timeout to keep reads responsive (TIMEOUT_S = 0.15).

  - Before sending any commands, the script drains the serial buffer to remove leftover startup messages or partial lines from previous runs, which helps prevent parsing errors later when converting data to CSV.

  - sync_to_tuning_ui() forces the system into a known state by:

    - Sending a couple carriage returns

    - Sending the h key to request the tuning banner/menu

    - Waiting until the banner text and prompt are detected

- Automatically setting controller gains and setpoint

  - set_gains_and_setpoint() drives the tuning UI exactly like a user would:

    - Sends k to enter gain-entry mode

    - Waits for “Enter proportional gain …” then sends kp + Enter

    - Waits for “Enter integral gain …” then sends ki + Enter

    - Sends s to enter setpoint mode

    - Sends setpoint_mm_s + Enter

  - Each step uses read_until_any() to block until the expected prompt appears (or times out), which prevents the script from “typing ahead” faster than the MCU can respond. This way we do not have to use delays as previously tried which greatly speed up the testing process.

- Running step response tests (left then right)

  - collect_step_response_single_side() performs one test on either l or r:

    - Sends g to enter the data-collection UI

    - Waits for confirmation that the user task is active

    - Sends l (left) or r (right) to start a step response

    - Waits for “Data collection complete …”

    - Waits until the prompt returns, meaning all CSV data has printed

    - Sends h to return cleanly back to the tuning interface

  - As configured, it runs left then right, so each dataset stays separate and legends/plots are clean.

  - The serial output is collected as raw text blocks (including headers, menus, and CSV). The script also saves a _raw.txt file containing the full unfiltered serial transcript for debugging if parsing fails.

    - parse_csv_from_text() is intentionally conservative:

      - It ignores everything until it sees the CSV header line containing Time [us]

      - Then it reads only lines of the form: <time>,<value>

      - It stops when it sees the footer (“Commands:” or dashed lines)

    - This ensures the script doesn’t accidentally treat UI banners or status messages as data.

- Unit conversion

  - Since the MCU prints time in microseconds, the script converts to seconds via:

    - t_s = t_us × 1e-6

  - That produces time vectors compatible with typical control plots and performance metrics.

- Output files and organization

  - The script creates a timestamped “Collection Log” directory and saves:

    - MM_DD_HH_MM_SS_left.csv

    - MM_DD_HH_MM_SS_right.csv

    - MM_DD_HH_MM_SS.png (combined plot with legend)

    - MM_DD_HH_MM_SS_raw.txt (full serial transcript)

    - Each CSV includes metadata headers (setpoint, Kp, Ki) so datasets remain traceable even when collected in bulk.

- Plotting and performance metrics

  - save_plot() plots left and right step responses on the same axes and draws a dashed horizontal line at the commanded setpoint. This provides an immediate visual check of rise behavior, overshoot, and steady-state tracking.

  - compute_step_metrics() estimates basic performance quantities per motor:

  - steady-state value y_ss (average of the last chunk of samples)

  - peak value y_pk and time of peak t_pk

  - percent overshoot relative to setpoint: %OS=ypk-SPSP×100%

  - These metrics are printed to the console for quick tuning feedback.

# Task Scheduler

The program is organized as a set of cooperative, generator based tasks scheduled at a fixed rate. Each task has a single responsibility and communicates with other tasks using Share and Queue objects instead of direct calls, which keeps the system modular and prevents tight coupling between motor control, sensing, and the user interface.

- task_motor

  - Period: ~10 ms (100 Hz)

  - Priority: High

  - Reads the encoder every cycle and computes measured position and velocity, and publishes them through shares.

  - Applies the effort command to the motor driver.

  - Optionally logs data into queues for later printing.

  - The motor task must run consistently and predictably, so it should not be blocked by user I/O or printing.

- task_control / task_control_loop

  - Period: ~10 ms (same as motor task)

  - Priority: High (equal to motor task, or slightly lower but still above UI)

  - Closed-loop controller that computes the effort command.

  - When enabled, it reads the current setpoint and gains, reads measured speed (omega_meas), runs the PID math, and writes an effort command.

  - When disabled, it forces effort to zero.

  - It resets internal controller state on the rising edge of enable to prevent integrator windup and derivative spikes.

- task_user (data collection UI)

  - Period: ~50–100 ms

  - Priority: Low

  - Reads serial commands (l, r, b, x, h).

  - Starts a data collection run by setting “go” flags for the motors and clearing the appropriate queues.

  - After the buffers fill, it prints the stored time/data pairs as CSV

  - Serial printing is slow and unpredictable, so isolating it prevents it from interfering with real-time sensing/control.

  - task_tuning_ui (Implemented into task user)

    - Runs only when start_user == 0.

    - Updates Kp, Ki, setpoint.

    - Starts the data-collection UI by setting start_user = 1.

Table 1. Table of Shares and Queues

| Variable Name | Data Stored | Written By | Read By | Purpose |
| --- | --- | --- | --- | --- |
| motorLGo | Left motor enable flag (0/1) | task_user, task_tuning_ui | task_motor, task_control | Enables or disables the left motor and its control loop during testing. |
| motorRGo | Right motor enable flag (0/1) | task_user, task_tuning_ui | task_motor, task_control | Enables or disables the right motor and its control loop during testing. |
| setpoint | Desired reference speed (counts/s) | main, task_tuning_ui | task_control, task_user | Stores the user-entered speed command used by the controller. |
| Kp | Proportional gain | main, task_tuning_ui | task_control, task_user | Allows real-time tuning of proportional control action. |
| Ki | Integral gain | main, task_tuning_ui | task_control, task_user | Allows real-time tuning of integral control action. |
| omegaL | Left wheel velocity (counts/s) | task_motor | task_control, task_user | Transfers left encoder velocity measurement to the control loop and UI. |
| omegaR | Right wheel velocity (counts/s) | task_motor | task_control, task_user | Transfers right encoder velocity measurement to the control loop and UI. |
| effortL | Left motor effort command (%) | task_control | task_motor | Communicates the controller output to the left motor driver. |
| effortR | Right motor effort command (%) | task_control | task_motor | Communicates the controller output to the right motor driver. |
| start_user | UI ownership flag (0 = tuning UI, 1 = data UI) | main, task_tuning_ui | task_user | Determines which UI task has control of the serial interface. |
| posL | Left encoder position (counts) | task_motor | main, task_user | Provides left position data for logging and plotting. |
| posR | Right encoder position (counts) | task_motor | main, task_user | Provides right position data for logging and plotting. |
| dataValuesL | Left motor speed samples (counts/s) | task_motor | task_user | Buffers left motor step-response velocity data for CSV printing. |
| timeValuesL | Left motor timestamps (µs) | task_motor | task_user | Stores elapsed time values aligned with left speed samples. |
| dataValuesR | Right motor speed samples (counts/s) | task_motor | task_user | Buffers right motor step-response velocity data for CSV printing. |
| timeValuesR | Right motor timestamps (µs) | task_motor | task_user | Stores elapsed time values aligned with right speed samples. |

Table 2. Table of Shares and Queues

| Task name | Period (ms) | Priority |
| --- | --- | --- |
| task_motor_L | 10 | 2 |
| task_control_L | 10 | 2 (or 1) |
| task_motor_R | 10 | 2 |
| task_control_R | 10 | 2 (or 1) |
| task_user | 50–100 | 0 |
| task_tuning_ui | 50–100 | 0 |

All tasks ran on time according to the scheduler profiler, with the motor and control tasks consistently meeting their assigned periods and showing no deadline misses. These tasks were given higher priority and shorter periods to ensure stable closed-loop control and accurate encoder updates. The user interface tasks were assigned lower priority and longer periods since serial communication and printing are slower and not time-critical. This priority structure successfully protected the real-time control loop from being disrupted by UI activity, producing the intended behavior of reliable motor control alongside functional data collection and printing.

While planning the overall software architecture, state transition diagrams and task interaction diagrams were created to visualize how the system operates and how different tasks communicate with one another. The state diagrams (Figures 1, 2, and 3) illustrate the internal behavior of individual tasks, showing how each task transitions between states in response to user input, control flags, and system conditions. The task interaction diagram provides a higher-level view of the system, highlighting how data is passed between tasks through shared variables and queues under the cooperative multitasking scheduler. Together, these diagrams were used to guide implementation, verify correct task coordination, and ensure that real-time motor control was not blocked by user interaction or data logging.

Figure 1. Task Diagram

Figure 2. Task Motor FSM

Figure 3. Task User FSM

Figure 3. Task Closed Loop FSM

# Plots

Figure 3. Step Response: Test 1

Figure 4. Step Response: Test 2

The step response plots generally demonstrate the expected behavior of a closed-loop second-order system. In both tests, the motors exhibit a clear transient response characterized by an initial rise, overshoot, and damped oscillations before settling toward a steady-state value. This confirms that the proportional–integral controller is producing the anticipated dynamic behavior.

However, a consistent discrepancy is observed between the commanded setpoint and the achieved steady-state motor speed. In both cases, the steady-state velocity settles at approximately half of the desired setpoint value. This suggests a systematic scaling issue rather than random error. A likely cause may be a unit conversion mismatch, or an error in how the setpoint is interpreted relative to the measured velocity. However, I believe that our batteries were low on power unfortunately which lead to our half-powered system. Despite this steady-state offset, the overall response shape clearly reflects second-order dynamics with visible overshoot and damping, meeting the qualitative behavior expected for the system.

References:

ME 405 Class Notes

Chat GPT

# Main

from motor_driver import motor_driver

from encoder import encoder

from task_motor import task_motor

from task_user import task_user, task_tuning_ui

from task_share import Share, Queue

from cotask import Task, task_list

from task_control_loop import task_control

from gc import collect

import pyb

# -----------------------------

# USB

# -----------------------------

ser = pyb.USB_VCP()

ser.write(b"\r\n*** main.py started ***\r\n")

pyb.delay(50)

# -----------------------------

# Hardware setup

# -----------------------------

PWM_FREQ = 20000

tim4 = pyb.Timer(4, freq=PWM_FREQ)

pwmL = tim4.channel(4, pyb.Timer.PWM, pin=pyb.Pin('PB9'))

pwmR = tim4.channel(3, pyb.Timer.PWM, pin=pyb.Pin('PB8'))

motorL = motor_driver(pwmL, 'PC0', 'PC1')

motorR = motor_driver(pwmR, 'PC2', 'PC3')

encL = encoder(2, 'PA0', 'PA1')

encR = encoder(1, 'PA8', 'PA9')

# -----------------------------

# Shares

# -----------------------------

motorLGo = Share("B", name="Left Enable")

motorRGo = Share("B", name="Right Enable")

setpoint = Share("f", name="Setpoint")

Kp = Share("f", name="Kp Gain")

Ki = Share("f", name="Ki Gain")

omegaL = Share("f", name="Omega L")

omegaR = Share("f", name="Omega R")

effortL = Share("h", name="Left Effort")

effortR = Share("h", name="Right Effort")

start_user = Share("B", name="Start User Task")

posL = Share("l", name="Pos L")

posR = Share("l", name="Pos R")

# -----------------------------

# Initial values

# -----------------------------

motorLGo.put(0)

motorRGo.put(0)

setpoint.put(1200.0)   # counts/sec

Kp.put(0.02)

Ki.put(0.0)

effortL.put(0)

effortR.put(0)

start_user.put(0)   # tuning UI will set this to 1

# -----------------------------

# Queues

# -----------------------------

dataValuesL = Queue("f", 100, name="Left Data Buffer")

timeValuesL = Queue("L", 100, name="Left Time Buffer")

dataValuesR = Queue("f", 100, name="Right Data Buffer")

timeValuesR = Queue("L", 100, name="Right Time Buffer")

# -----------------------------

# Motor tasks

# -----------------------------

leftMotorTask = task_motor(

motorL,

encL,

motorLGo,

effortL,

posL,

omegaL,

dataValuesL,

timeValuesL

)

rightMotorTask = task_motor(

motorR,

encR,

motorRGo,

effortR,

posR,

omegaR,

dataValuesR,

timeValuesR

)

# -----------------------------

# Control tasks

# -----------------------------

def cl_left():

yield from task_control(

motorLGo,

setpoint,

Kp,

Ki,

omegaL,

effortL

)

def cl_right():

yield from task_control(

motorRGo,

setpoint,

Kp,

Ki,

omegaR,

effortR

)

# -----------------------------

# UI tasks

# -----------------------------

tuningTask = task_tuning_ui(

start_user,

setpoint,

Kp,

Ki,

ser=ser

)

userTask = task_user(

start_user,

motorLGo,

motorRGo,

dataValuesL,

timeValuesL,

dataValuesR,

timeValuesR,

setpoint,

Kp,

Ki,

ser

)

# -----------------------------

# Scheduler

# -----------------------------

task_list.append(Task(leftMotorTask.run,  name="Left Motor",  priority=1, period=50))

task_list.append(Task(rightMotorTask.run, name="Right Motor", priority=1, period=50))

task_list.append(Task(cl_left,  name="Left Control",  priority=2, period=50))

task_list.append(Task(cl_right, name="Right Control", priority=2, period=50))

task_list.append(Task(tuningTask.run, name="Tuning UI", priority=0, period=50))

task_list.append(Task(userTask.run,   name="User",      priority=0, period=50))

collect()

# -----------------------------

# RUN

# -----------------------------

while True:

try:

task_list.pri_sched()

except KeyboardInterrupt:

motorL.disable()

motorR.disable()

print("Program Terminated")

break

*Files cotask and task_share from John Ridgley also required

Please see the main code below as evidence that our program meets the objectives described above. Included is the output from a representative test run, along with a copied excerpt of the serial scroll back from our user interface. While the raw terminal output may appear cluttered, it reflects the received (RX) data being echoed from PuTTY into a text log, which was then processed and plotted. For clarity, a corresponding CSV file generated from the same test is also provided below.

Starting data-collection UI...

User task active.

Commands: l=left, r=right, b=both, x=stop, h=menu

L

Setpoint: 300.0 mm/s

Kp:       0.15 (%·s/mm)

Ki:       0.01 (%/mm)

50890,0.0

101258,91.79771

150885,180.6305

201264,183.8578

250934,144.4083

301393,100.6694

351071,93.2318

400772,108.2387

451230,130.2729

500952,132.2146

551428,124.3169

601162,120.154

650908,117.136

701403,124.2603

751158,126.1109

800920,126.0983

851431,124.2209

901199,126.0805

950972,126.0679

1001497,124.1889

1051281,126.0375

1101068,129.0306

1150864,132.0181

1201420,127.0754

1251234,128.9658

1301097,128.8391

1350931,131.9174

1401518,129.9432

1451365,128.8856

1501222,131.8459

1551083,131.8354

1600949,134.8262

1650818,131.8089

1701440,129.8508

1751318,134.7857

1801202,134.7748

1851090,134.7559

1900983,131.7534

1950877,134.7478

2001525,138.6303

2051430,137.7091

2101344,134.6884

2151262,134.6803

2201179,134.6803

2251105,137.6484

2301072,137.5355

2351018,137.6098

2400964,137.5933

2450910,137.5961

2500865,137.5685

2551591,138.4445

2601574,137.4942

2651561,140.4691

2701554,137.4639

2751551,137.4529

2801552,140.4298

2851561,143.3976

2901568,143.4005

2951581,140.3961

3001597,137.4007

3051619,140.3708

3100900,139.4586

3150929,143.3345

3200960,143.3288

3250998,140.3288

3301086,140.1831

3351136,143.2773

3401190,143.2658

3451245,146.2505

3501310,143.2343

3551374,143.2372

3601446,143.2143

3651518,143.2143

3701599,146.1746

3750935,145.3538

3801021,146.1571

3851113,143.1543

3901208,146.1308

3951311,146.1104

4001416,143.1171

4051523,146.0987

4101635,146.0812

4151002,148.2888

4201123,146.0579

4251246,143.0629

4301413,145.924

4351548,148.9941

4401689,148.9733

4451089,148.1988

4501238,145.9676

4551391,145.9618

4601547,148.9317

4650964,148.1388

4701132,148.8991

4751299,151.871

4801475,148.8694

4851653,148.8723

4901091,151.1009

4951276,148.8397

5001469,151.7953

Starting data-collection UI...

User task active.

Commands: l=left, r=right, b=both, x=stop, h=menu

R

Setpoint: 300.0 mm/s

Kp:       0.15 (%·s/mm)

Ki:       0.01 (%/mm)

51046,0.0

100824,95.88602

150604,189.0676

201141,183.2866

250928,138.0327

300726,96.00735

350531,92.98656

401097,121.1457

450925,140.9287

500756,137.9081

550592,122.9131

601188,115.1633

651051,122.849

700914,128.8365

750783,128.8184

800653,128.8107

850530,125.805

901199,129.7355

951086,128.7719

1000973,128.7745

1050866,131.7508

1100764,128.7487

1150667,131.7218

1200579,128.7074

1251239,132.7076

1301158,128.6894

1351085,131.6611

1401009,131.669

1450945,131.6452

1500881,128.6456

1550836,131.5925

1600789,134.586

1650745,131.5846

1700729,131.5399

1750716,134.4917

1800707,134.481

1850704,134.4675

1900748,131.3506

1950752,134.4487

2000761,134.4353

2050771,137.4145

2100785,140.3905

2150804,134.4084

2200827,134.3976

2250861,134.3654

2300898,140.326

2350937,137.3403

2400978,137.3321

2451027,137.3101

2501075,137.3101

2551135,140.2671

2601194,137.2827

2651261,140.2419

2701327,140.2503

2750654,136.2913

2800734,140.2111

2850814,143.1943

2900944,143.0458

2951040,140.1663

3001137,140.1607

3051235,140.1551

3101335,140.1523

3150699,145.2743

3200810,146.0842

3250932,143.0743

3301055,140.0852

3351185,143.0486

3401316,146.0259

3450707,145.192

3500843,143.0286

3550985,140.0405

3601129,145.9851

3651280,142.9859

3700691,148.1568

3750850,145.9473

3801013,142.9574

3851185,145.9007

3901394,145.799

3950826,145.0715

4001009,145.8775

4051198,145.8542

4101385,148.8397

4150836,145.0217

4201034,148.8041

4251244,148.7715

4300708,147.995

4350922,148.7715

4401138,145.7671

4451362,145.7555

4500842,150.9696

4551073,148.7093

4601309,151.6684

4650799,150.9391

4701043,148.6709

4751290,145.6859

4800793,150.8994

4850391,149.513

4900639,154.5931

4950992,151.3761

5000599,150.6286